COAL Lab-06

Load & Store Instruction Assembly Code

------------------------------------------------------

Name: Saad Nisar Butt

Reg. no: cs211246

Class: BSCS-3C-1

------------------------------------------------------

Literature Review:

Load Instructions:

Load instructions are used to move data from memory to registers (before operation). Loads are encoded in the I-type format. The effective byte address is obtained by adding register rs1 to the sign-extended 12-bit offset. Loads copy a value from memory to register rd. The assembly representation for load instructions are:

lw (destination_register), (offset)(source_register)

or

lw (rd), offset(rs1)

The LW instruction loads a 32-bit value from memory into rd. LH loads a 16-bit value from memory, then sign-extends to 32-bits before storing in rd. LHU loads a 16-bit value from memory but then zero extends to 32-bits before storing in rd. LB and LBU are defined analogously for 8-bit values.

Store Instructions:

Store instructions are used to move data from registers to memory (after operation). Stores are encoded in the S-type format. The effective byte address is obtained by adding register rs1 to the sign-extended 12-bit offset. Stores copy the value in register rs2 to memory.. The assembly representation for store instructions are:

sw (source_register_2), (offset)(source_register_1)

or

sw (rs2), offset(rs1)

The SW instruction stores a 32-bit value from the low bits of register rs2 to memory. SH stores a 16-bit value from the low bits of register rs2 to memory. SB stores a 8-bit value from the low bits of register rs2 to memory.

Lab Exercise 1:

Task:

Run the below assembly code on Venus Simulator

li s0, 0x12345678 # Data to be store

li s1, 0x00000020 # memory address

sb s0, 0x0(s1)

sh s0, 0x4(s1)

sw s0, 0x8(s1)

sb s0, 0x0(s1)

| 0000000 | 01000 | 01001 | 000 | 00000 | 0100011 |
| --- | --- | --- | --- | --- | --- |

Machine Code:  0000 0000 1000 0100 1000 0000 0010 0011

Hexadecimal Code:  00848023

sh s0, 0x4(s1)

| 0000000 | 01000 | 01001 | 001 | 00100 | 0100011 |
| --- | --- | --- | --- | --- | --- |

Machine Code:  0000 0000 1000 0100 1001 0010 0010 0011

Hexadecimal Code:  00849223

sw s0, 0x8(s1)

| 0000000 | 01000 | 01001 | 010 | 01000 | 0100011 |
| --- | --- | --- | --- | --- | --- |

Machine Code:  0000 0000 1000 0100 1010 0100 0010 0011

Hexadecimal Code:  0084A423

Venus Simulation:

Lab Exercise 2:

Task:

Run the below assembly code on Venus Simulator

lb t0, 0x0(x0)

lbu t1, 0x4(x0)

lh t2, 0x8(x0)

lhu s0, 0xC(x0)

lw s1, 0x10(x0)

lb t0, 0x0(x0)

| 000000000000 | 00000 | 000 | 00101 | 0000011 |
| --- | --- | --- | --- | --- |

Machine Code:  0000 0000 0000 0000 0000 0010 1000 0011

Hexadecimal Code:  00000283

lbu t1, 0x4(x0)

| 000000000100 | 00000 | 100 | 00110 | 0000011 |
| --- | --- | --- | --- | --- |

Machine Code:  0000 0000 0100 0000 0100 0011 0000 0011

Hexadecimal Code:  00404303

lh t2, 0x8(x0)

| 000000001000 | 00000 | 001 | 00111 | 0000011 |
| --- | --- | --- | --- | --- |

Machine Code:  0000 0000 1000 0000 0001 0011 1000 0011

Hexadecimal Code:  00801383

lhu s0, 0xC(x0)

| 000000001100 | 00000 | 101 | 01000 | 0000011 |
| --- | --- | --- | --- | --- |

Machine Code:  0000 0000 1100 0000 0101 0100 0000 0011

Hexadecimal Code:  00C05403

lw s1, 0x10(x0)

| 000000010000 | 00000 | 010 | 01001 | 0000011 |
| --- | --- | --- | --- | --- |

Machine Code:  0000 0001 0000 0000 0010 0100 1000 0011

Hexadecimal Code:  01002483

Venus Simulation

InLab Tasks:

Task 1:

Write down a simple assembly program to add, and subtract two integer numbers and store their result into different memory locations. Stimulate the code on Venus.

RISC-V Assembly Code:

addi x5, x0, 4 # storing 4 in register x5

addi x6, x0, 6 # storing 6 in register x6

add s0, x5, x6 # addition

sub s1, x6, x5 # subtraction

addi x20, x20, 0x20 # setting memory’s base address

sw s0, 0x0(x20) # storing s0’s added value in memory

sw s1, 0x4(x20) # storing s1’s subtracted value in memory

Venus Simulation:

Task 2:

Write down a simple assembly program to load the contents from memory into registers and perform the logical operations on them. Stimulate the code on Venus.

RISC-V Assembly Code:

addi x20, x20, 0x20 # setting memory’s base address

lw s2, 0x0(x20) # loading data from memory into register

lw s3, 0x4(x20) # loading data from memory into register

and s5, s2, s3 # ANDing data loaded from memory

or s6, s3, s2 # Oring data loaded from memory

Venus Simulation:

* -------------------------------------- *